### CEE LP-200B Moving Head Beam Light

## Technical parameter：

Lamp： YODN MSD 200 R5 (OSRAM or PHILIPS selectable)

Lamp Life： 2000 H

Lumen：  8300 lm

Color Temperature： 8500K

Beam Angle： Parallel Beam Angle 0－3.9°

Power Supply： AC 100V-240V，50 / 60Hz

Power Consumption：250W

Control Signal： DMX-512 (1990)

Channel：  16CH standard mode / 20ch extended mode

Movement Range：  Pan 540°, 16Bit resolution, automatic position correction

Tilt 270°, 16Bit resolution, automatic position corr Mechanical linear dimmer 0－100%

Color Wheel： 1 fixed color wheel（14 color filters + white）, linear color conversion, semi-color effect, rainbow effect in both directions

Gobo Wheel： 17 fixed gobos + white, with gobo shake function at variable speed / rotating effect in both directions

Prism： Independent 16-facet prism, prism locating function and rotating in both directions at variable speed

Focus： Incredibly flexible linear focus

Strobe： variable strobe 1-15 flashes per second, selectable random or pulse strobe

Atomization： 0-100% linear atomization

Environment： -20℃～40℃, well ventilated

Protection Class： IP20

Size： 398 x 330 x 540mm

Weight： 17.3Kg